PRODUCT REQUIREMENTS DOCUMENT

Tamil Nadu AI Floor Plan Generator

Version 1.0  |  March 2026  |  Academic Review Build

Project Root: D:\layout_project

Primary Tool: Claude Code + Codex (Vibe Coding)

Compliance: NBC 2016 | TNCDBR 2019 | CMDA | DTCP | Vastu | Baker Principles

# 1. Problem Statement

Residential floor plan design in Tamil Nadu currently requires an architect, costs ₹10,000–50,000 per plan, and takes 2–4 weeks. Small homeowners — especially EWS, LIG, and MIG plot owners — cannot afford professional design services and often build without proper plans, resulting in NBC code violations, poor circulation, Vastu non-compliance, and buildings that fail DTCP approval.

Existing AI floor plan tools (Maket.ai, etc.) are trained on Western housing typologies and produce layouts that do not reflect Tamil Nadu residential norms — wrong room adjacency, no Vastu consideration, no Baker principle integration, no CMDA/DTCP setback compliance, and no climate-responsive design for Tamil Nadu's varied climate zones.

This project generates professional, regulation-compliant 2D floor plans specific to Tamil Nadu plot sizes, BHK configurations, climate zones, and regional building codes — in under 5 seconds, for free.

# 2. Target Users

| User Type | Use Case | Key Need |
| --- | --- | --- |
| Homeowner (EWS/LIG) | First-time construction on TNHB/DTCP plot | Valid NBC-compliant plan without paying architect |
| Homeowner (MIG/HIG) | Renovation or new construction guidance | Climate-responsive layout with material recommendations |
| Architecture Student | Reference and learning tool | See how rules map to geometry for TN plots |
| Junior Architect | Quick first-draft generation | Starting point that already satisfies compliance |
| DTCP Approval Consultant | Pre-check before formal submission | Setback and NBC compliance verification |
| Academic Reviewer | Project evaluation | Demonstrable AI + compliance + professional output quality |

# 3. Scope

## 3.1 In Scope

- Ground floor plan generation for all Tamil Nadu DTCP/CMDA/TNHB plot sizes

- First floor plan (same footprint, modified room program for G+1)

- Half floor / terrace plan annotation

- BHK configurations: 1BHK through 4BHK per plot size constraints

- North, South, East, West facing plots

- All 38 Tamil Nadu districts with district-specific climate and soil data

- NBC 2016 room area and width compliance

- TNCDBR 2019 and CMDA/DTCP setback rules

- Vastu shastra directional compliance (room placement by compass zone)

- Baker principles: thermal mass, passive cooling, cross-ventilation

- Climate-responsive window and ventilator placement

- Soil type influence on foundation material recommendation

- District-specific material recommendations (brick type, roofing, flooring)

- 7-metric scoring: Vastu, NBC, Circulation, Adjacency, Climate, Baker, Overall

- SHAP explainability: which features drove the validity score

- PNG export at 150 DPI (A2 print quality)

- DXF export for CAD import (ezdxf wall-centric format)

- Streamlit web app with form input and plan preview

## 3.2 Out of Scope (Version 1.0)

- Structural grid design and load-bearing wall calculation

- Electrical wiring layout

- Plumbing routing and pipe sizing

- Staircase design beyond space allocation

- Multi-unit apartment layouts

- Cost estimation

- 3D model generation

- BIM integration (IFC export)

- Parking and vehicle circulation design

- Accessibility (wheelchair) compliance — future version

# 4. Supported Plot Sizes & BHK Matrix

The following plot sizes from the official Tamil Nadu plot classification table are supported. BHK configuration is constrained by plot area — the engine will not generate a 3BHK on an EWS plot.

EWS & LIG Plots (1–2 BHK only)

| Plot Size (m) | Area (sqm) | Authority | Max BHK | Layout Type |
| --- | --- | --- | --- | --- |
| 5×9 | 45 | TNHB | 1BHK | Narrow |
| 6×9 | 54 | TNHB | 1BHK | Square |
| 6×12 | 72 | DTCP/TNHB | 1BHK | Square |
| 7.5×10 | 75 | DTCP | 2BHK | Square |
| 7×12 | 84 | DTCP | 2BHK | Square |
| 7.5×12 | 90 | DTCP/TNHB | 2BHK | Square |

Standard Residential Plots (2–3 BHK)

| Plot Size (m) | Area (sqm) | Authority | Max BHK | Layout Type |
| --- | --- | --- | --- | --- |
| 9×12 | 108 | DTCP | 2BHK | Square — Most common TN |
| 10×15 | 150 | DTCP/CMDA | 3BHK | Square |
| 12×15 | 180 | DTCP/CMDA ★ | 3BHK | Square — Most common TN |
| 12×18 | 216 | CMDA | 3BHK | Square |
| 9×20 | 180 | DTCP/CMDA | 3BHK | Narrow |
| 10×20 | 200 | CMDA/DTCP | 3BHK | Narrow |

Premium & Large Plots (3–4 BHK)

| Plot Size (m) | Area (sqm) | Authority | Max BHK | Layout Type |
| --- | --- | --- | --- | --- |
| 15×15 | 225 | CMDA/DTCP ★ | 4BHK | Square |
| 15×20 | 300 | CMDA | 4BHK | Square |
| 18×20 | 360 | CMDA | 4BHK | Square |
| 20×25 | 500 | CMDA | 4BHK | Square |

# 5. Room Placement Rules (Architectural Law)

These rules are non-negotiable. They reflect actual Tamil Nadu residential practice as observed in reference plans and NBC/TNCDBR compliance requirements. The engine must produce layouts that match these rules — not approximate them.

## 5.1 Zone Layout (North-Facing Reference Plan)

| Band | Zone | Rooms | Position |
| --- | --- | --- | --- |
| Band 1 (north/top) | Entry | Verandah (full width, 1.5m depth) | Road-facing, thermal buffer |
| Band 2 | Public | Living + Dining | Living left 55–65%, Dining right 35–45% |
| Band 3 | Service | Kitchen (NE) + Utility + Toilet Common (NW) + Pooja (NE corner) | Service cluster, plumbing wall shared |
| Band 4 (south/bottom) | Private | Master Bedroom (SW) + Toilet Attached + Bedroom 2/3/4 | Bedrooms staggered, not in row |

## 5.2 Mandatory Adjacency Rules

- Verandah must share full north wall — no room may face the road except verandah

- Living must be directly accessible from verandah (shared wall or max 0.5m gap)

- Kitchen must be adjacent to Utility (shared wall)

- Kitchen must be adjacent to Dining (max 1.5m separation)

- Master Bedroom must share a wall with Toilet Attached

- Common Toilet must never be adjacent to Kitchen or Dining

- Pooja room must be in NE corner of the plan (Vastu mandatory)

- Corridor width minimum 1.0m between public and private zones

## 5.3 NBC 2016 Minimum Room Dimensions

| Room | Min Area (sqm) | Min Width (m) | Notes |
| --- | --- | --- | --- |
| Master Bedroom | 9.5 | 2.8 | NBC Table 5 |
| Bedroom 2/3/4 | 7.5 | 2.4 | NBC Table 5 |
| Living Room | 10.0 | 2.4 | NBC hard floor; 3.0m preferred |
| Dining Room | 6.0 | 2.1 | NBC Table 5 |
| Kitchen | 4.5 | 1.8 | TNCDBR 2019 |
| Toilet Attached | 2.3 | 1.1 | NBC minimum |
| Common Toilet | 2.3 | 1.1 | NBC minimum |
| Utility | 2.5 | 1.2 | Project standard |
| Verandah | net_width × 1.5m | Full plot width | Standard TN practice |
| Pooja | 1.5 | 1.0 | Vastu minimum |

## 5.4 Wall Thickness Standards

- Exterior walls: 230mm (one brick length) — structural, thermal mass

- Interior partition walls: 115mm (one brick width) — acoustic separation

- Wall rendering in DXF/PNG must show actual wall polygon (not just lines)

- Door openings: cut through wall polygon with swing arc shown

- Window openings: cut through wall polygon with sill depth shown

# 6. Multi-Floor Specification

## 6.1 Ground Floor (G)

Standard band layout as described in Section 5. All primary living, service, and private rooms on ground floor. Staircase allocated in service band (Band 3), minimum 1.0m × 2.5m space.

## 6.2 First Floor (G+1)

- Footprint identical to ground floor (same structural walls)

- Verandah becomes a terrace/balcony or additional bedroom

- Living/Dining becomes additional bedrooms or study/prayer room

- Kitchen becomes a dry kitchen or storage

- Additional bathrooms aligned above ground floor wet areas (plumbing stack)

- Staircase landing at top of stair — same position as ground floor

- BHK count increases by 1–2 bedrooms on first floor

- Flat terrace slab shown in plan with parapet wall annotation

## 6.3 Half Floor / Terrace

- Shown as plan view with: water tank position, stairhead enclosure, solar panel zone

- Parapet height 1.0m minimum (NBC safety requirement)

- Terrace slope direction annotated for drainage

# 7. Climate & Regional Adaptation

## 7.1 Tamil Nadu Climate Zones

| Zone | Districts (examples) | Key Design Response |
| --- | --- | --- |
| Hot & Humid | Chennai, Kanchipuram, Vellore, Cuddalore | Max cross-ventilation, N+S windows, shaded verandah, terracotta tiles |
| Hot & Dry | Madurai, Tirunelveli, Ramanathapuram, Thoothukudi | Thermal mass walls, small east/west openings, courtyard if possible, mud brick |
| Warm & Humid | Coimbatore, Erode, Salem, Namakkal | Wire-cut brick, moderate openings, east-facing kitchen |
| Composite | Trichy, Thanjavur, Pudukkottai, Dindigul | Balanced approach, cavity walls on west, verandah on prevailing wind side |
| Cold (Hills) | Nilgiris, Kodaikanal zone | South-facing max glazing, thick stone walls, minimal north openings |

## 7.2 Window Placement Rules by Climate

- Hot & Humid: windows on N and S walls preferred, avoid W walls, ventilators in all wet rooms

- Hot & Dry: small openings on E and W, larger on N, no south windows on bedrooms

- All zones: kitchen must have cross-ventilation (window + exhaust ventilator)

- All zones: toilet/bathroom must have ventilator on exterior wall

## 7.3 Soil Type Influence on Foundation Recommendation

| District Group | Predominant Soil | Foundation Recommendation |
| --- | --- | --- |
| Chennai, Kanchipuram coast | Black cotton + alluvial | Raft foundation or under-reamed pile; avoid strip footing |
| Coimbatore, Salem inland | Red laterite + hard rock | Strip footing on rock, 600mm depth; RCC frame |
| Madurai, Tirunelveli | Sandy loam + black cotton patches | Strip footing with anti-heave measures; DPC at 450mm |
| Nilgiris, hill stations | Rocky terrain + clay | Rock-cut foundation; rubble stone masonry wall base |
| Delta districts (Thanjavur, Nagapattinam) | Deep alluvial | Pile foundation; floating slab for lightweight structures |

## 7.4 Baker Principles Integration

Laurie Baker's cost-effective and climate-responsive principles are applied as follows:

- Rat-trap bond brickwork recommended where applicable (20% brick saving, better insulation)

- Filler slab roofing for spans up to 4.5m (terracotta pots or brick panels in slab)

- Verandah depth minimum 1.5m — acts as climate buffer, reduces direct solar gain

- Jali (perforated brick screen) recommended for west-facing walls in hot-dry zones

- Natural ventilation stack effect: service band (kitchen/utility) positioned for heat exhaust

- Minimal corridor principle: rooms open directly to each other where possible

- Local material preference: wire-cut bricks (Coimbatore), country bricks (rural), stabilised mud blocks (Nilgiris)

# 8. Scoring System (7-Metric)

| Score | Weight | What It Measures | Pass Threshold |
| --- | --- | --- | --- |
| score_vastu | 15% | Room compass zone compliance (kitchen SE/NE, pooja NE, master SW) | ≥ 0.80 |
| score_nbc | 25% | Room areas and widths vs NBC 2016 Table 5 minimums | ≥ 0.85 |
| score_circulation | 20% | Corridor exists, no room requires passing through another, min 1.0m width | ≥ 0.80 |
| score_adjacency | 20% | Mandatory adjacency rules satisfied (kitchen-utility, master-toilet, etc.) | ≥ 0.85 |
| score_climate | 10% | Window orientation matches district climate zone preference | ≥ 0.75 |
| score_baker | 10% | Baker principle features present (verandah depth, material selection) | ≥ 0.70 |
| score_overall | 100% | Weighted combination of all 6 scores; is_valid = 1 if overall ≥ 0.85 | ≥ 0.85 |

The ML constraint scorer (XGBoost, AUC 1.00) provides a learned validity probability in addition to the rule-based scores. Plans with score_valid < 0.4 trigger a retry with different seed (max 15 attempts).

# 9. Streamlit App Specification (Phase 6)

This is the primary deliverable for academic review. The app must be functional, professional, and demonstrate the full pipeline from user input to plan output.

## 9.1 Input Form

| Field | Type | Options / Constraints | Default |
| --- | --- | --- | --- |
| District | Selectbox | All 38 Tamil Nadu districts | Coimbatore |
| Plot Width (m) | Number input | 4.0 – 50.0m, step 0.5 | 12.0 |
| Plot Depth (m) | Number input | 8.0 – 40.0m, step 0.5 | 15.0 |
| Plot Facing | Radio | North / South / East / West | North |
| BHK Configuration | Selectbox | 1BHK / 2BHK / 3BHK / 4BHK (filtered by plot area) | 2BHK |
| Number of Floors | Radio | G / G+1 / G+2 | G |
| Vastu Compliance | Checkbox | Enable/disable Vastu zone enforcement | Enabled |
| Baker Principles | Checkbox | Enable/disable Baker cost-saving features | Enabled |
| Random Seed | Number input | 0 – 999 (optional; auto if blank) | Auto |

## 9.2 Validation Before Generation

- Plot area must support selected BHK — show warning if not (e.g., 6×9m cannot be 2BHK)

- Net buildable area must be above 30 sqm after setbacks — show error if not

- District must be selected before generation

## 9.3 Output Panel

- Floor plan PNG displayed inline (large, readable — A3 equivalent)

- Score panel: 7 scores shown as color-coded progress bars (green ≥ threshold, amber near, red below)

- SHAP explanation: top 5 features that most influenced the validity score

- Room area table: each room's generated dimensions vs NBC minimum

- Material recommendations panel: district-specific brick, roof, floor, foundation

- Baker principles panel: which principles were applied and why

- Soil note: foundation recommendation for selected district

- Download buttons: PNG (150 DPI), DXF (CAD), PDF report

## 9.4 Multi-Floor Display

- If G+1 selected: show ground floor and first floor plans side by side

- Floor selector tabs if G+2: Ground / First / Terrace

- Each floor shows its own score panel

## 9.5 UI Quality Standards

- Professional dark sidebar with project branding

- Floor plan displayed with: property boundary, setback lines, north arrow, scale bar, dimension annotations

- Wall thickness visible in plan (230mm exterior = 2px at scale, 115mm interior = 1px)

- Room fills with distinct colors matching the reference plan legend

- Furniture symbols inside rooms (bed, sofa, dining table, toilet fixture, kitchen platform)

- Door swing arcs shown

- Window gaps shown on exterior walls

- Room labels with dimensions (e.g., MASTER BEDROOM 4.2m × 3.2m)

# 10. Renderer Specification

## 10.1 Wall-Centric Drawing Standard

The renderer must produce plans that match the reference image quality — wall-centric, not room-fill based. This is the primary visual quality requirement.

- Walls drawn as filled solid polygons, not room border outlines

- Exterior walls: 230mm thick, dark gray fill (#2C2C2C), ANSI31 hatch pattern

- Interior walls: 115mm thick, medium gray fill (#555555)

- T-junction and L-junction cleanup using Shapely union before rendering

- Door openings cut through wall polygon — opening width from passage_dimensions DB

- Door swing arc drawn on correct side (inward swing default)

- Window openings cut through wall polygon — sill depth 150mm shown

- Column symbols at corners (100mm × 100mm filled squares)

## 10.2 Annotation Standards

- Room label: bold, centered, room name + dimensions (e.g., BEDROOM 2 / 3.6m × 3.0m)

- Dimension arrows: centerline style, 230mm or 115mm noted on wall thickness callouts

- Property boundary: dashed line with 'Property Boundary' label

- Setback dimensions: annotated with arrows

- North arrow: top-right corner

- Scale bar: bottom of plan

- Title block: plot size, BHK, facing, district, climate zone

## 10.3 Export Formats

| Format | Library | Use Case | Quality |
| --- | --- | --- | --- |
| PNG | matplotlib | Preview, sharing, print | 150 DPI, A2 size |
| DXF | ezdxf | CAD import, DTCP submission preparation | Wall polygons on A-WALL layer, rooms on A-ROOM layer |
| PDF | reportlab or matplotlib | Full report with scores and recommendations | A3 landscape |

# 11. Current Phase Status

| Phase | Description | Status | Notes |
| --- | --- | --- | --- |
| Phase 0 | 11 CSV seed files | Complete | NBC, Vastu, Baker, climate, materials, setbacks, adjacency, circulation |
| Phase 1 | SQLite DB (11 tables, 1,634 rows) | Complete | Indexed, validated |
| Phase 2 | Synthetic training data | Complete | 50,000 samples, 4,516 valid (9.0%), 157 columns, all TN plot sizes |
| Phase 3 | ML model training | Complete | XGBoost AUC 1.00, Keras dim model, SHAP explainer — retrained on correct data |
| Phase 4 | Engine (room placement + scoring) | Complete | Zone-based _place_rooms(), 15-attempt retry loop, score_valid > 0.5 for 12×15m |
| Phase 5 | Renderer (PNG + DXF output) | Partial | PNG working; wall-centric DXF renderer needed per Section 10 |
| Phase 6 | Streamlit App | Not Started | Primary deliverable — see Section 9 |

# 12. Known Limitations (Version 1.0)

| Limitation | Impact | Mitigation |
| --- | --- | --- |
| 1BHK small plots (EWS) have low scorer confidence | score_valid ~0.02 for 9×12 1BHK | Training data underrepresents this combination; will improve with more EWS samples |
| viol_bedroom_row fires occasionally | Bedrooms in exact row on some seeds | Stagger logic added; retry loop selects better seed |
| viol_plumbing_scattered ~52% in training data | Plumbing check aggressive | Fires on correct plans; model learned to tolerate this flag |
| Renderer not yet fully wall-centric | PNG shows room fills, not true wall polygons | Phase 6 renderer upgrade required — ezdxf double-line walls |
| No actual structural grid alignment | Wall positions not on 600mm modular grid | Future version — adds modular coordination |
| First floor uses same Keras model as ground floor | Room dims may not adapt for upper floor | Acceptable for V1; separate G+1 model in V2 |

# 13. Acceptance Criteria for Academic Review

The following criteria define 'done' for Version 1.0. All must pass before the project is considered complete.

## 13.1 Functional Criteria

- App accepts all inputs from Section 9.1 and generates a floor plan within 10 seconds

- Generated plan shows verandah on the correct road-facing wall for all 4 orientations

- Kitchen is never adjacent to Master Bedroom in any generated plan

- Attached toilet always shares a wall with Master Bedroom

- score_valid > 0.4 for at least 12 of 15 retry attempts for 12×15m 2BHK

- score_overall ≥ 0.85 for all standard plot sizes (9×12m and above)

- PNG export produces a plan visually comparable to the reference image in Section 13.3

- DXF export opens correctly in AutoCAD / LibreCAD with walls on separate layer

## 13.2 Compliance Criteria

- No generated plan has a room below NBC minimum area (score_nbc ≥ 0.85)

- All setbacks read from tn_setbacks table in DB — no hardcoded values

- Vastu score ≥ 0.80 for all plans with Vastu enabled

- Climate-zone window scores read from climate_data table in DB

## 13.3 Visual Quality Reference

The target visual quality is the reference plan (12×15m 2BHK, North-facing, Coimbatore) showing:

- Wall thickness annotated: 230mm exterior, 115mm interior

- Furniture symbols inside every room

- Dimension arrows on all rooms with measurements in metres

- Property boundary with setback callouts

- North arrow, scale, and title block

- Room color coding matching legend

- Door swing arcs and window gaps visible

# 14. Prompting Guide for Codex & Claude Code

## 14.1 Which Tool for Which Task

| Task | Use | Reason |
| --- | --- | --- |
| Multi-file reasoning (engine + generator alignment) | Claude Code (Opus 4.6) | Reads full codebase, plans across files |
| Single-file code generation (renderer, app.py) | Codex or Claude Code (Sonnet 4.6) | Fast, lower cost for mechanical generation |
| Debugging output (violation analysis) | Claude (this conversation) | Analysis and diagnosis before prompting |
| Data verification (parquet checks) | Codex locally | Runs Python directly on your machine |
| UI generation (Streamlit) | Claude Code (Sonnet 4.6) | Iterative UI best with Claude Code's file awareness |

## 14.2 Rules for Prompting Codex

- Always show a 5000-sample test output before running 50000

- Always show diffs before applying — never auto-apply patches

- Never ask Codex to fix violations without first diagnosing which specific lines cause them

- When a fix makes things worse, ask Codex to read the entire affected function before patching

- Keep FEATURE_COLS order identical between generate_training_data.py and engine.py — any mismatch breaks the scorer

- After any engine.py change, verify score_valid for 12×15m 2BHK before moving on

Tamil Nadu AI Floor Plan Generator — PRD v1.0

Compliance: NBC 2016 | TNCDBR 2019 | CMDA | DTCP | Vastu Shastra | Baker Principles